# JK FLIPFOP FREQUENCY ANSWR

Theory

- In Synchronous operation: All flip-flops receive the same clock, eliminating ripple-delay (all outputs change together) therefore the flip flops are synchronised

- Truncated sequence

  - They also know as Modulo counters (MOD-N): These counters out up to a predefined number N and then reset to Zero

  - They do not run the full 2n states

- So they can run the full states but sometimes we change that as in make the restart earlier

- As we observe the J-K flipflop we can see a few things which is the

  - All JKs are tied J=K=1 → they toggle on every clock edge.

  - The CLR pin has a little bubble (small circle) at its input.

    - That bubble means the input is active-LOW.

    - In simple terms to  activate the clear we have to input a low which is a zero

    - This will clear the flipflops (Restart them)

  - Now look at the and gate

    - It only gives us a 0 when all inputs are high

    - The output of the NAND gate is connected to clear of all the flipflops

    - So at the point where the NAND gate receives all 1s then it will restart the count

    - The inputs to the gate are Q1, Q3, Q4 when all of them are one thats when the count will restart

    - To get the limit in decimal we set the all Q pins to 0 except the one connected to the NAND gate do from Q4-Q0 (MSB-LSB)

    - 11010 which is 26 in decimal that is the limit

  The counter counts from 00000 (0) up to 11001 (25)

 At 11010 (26), the AND gate activates → drives CLR LOW → all FFs reset to 0.

  This makes the counter repeat every 26 cycles.

Frequency division

Therefore 
 output frequency= input clockmodulus

ANSWER

outputfrequency= input clockmodulus=10Mhz26=384615.3846=384.615x103=385Khz

- Two  input nand gate

- Q1& Q4 must be 1 to clear

- MSB-LSB 10010

- DEC = 18

ANSWER

outputfrequency= input clockmodulus=11Mhz18=611111.111=611.111103=611Khz